morph version 4.0   10 May 1993     image morphology program

by Richard Alan Peters II

Department of Electrical Engineering

Vanderbilt University School of Engineering

Nashville, TN 37235

(615) 322-7924

rap2@vuse.vanderbilt.edu

This software is freely redistributable if

the author's name and affiliation are included.

morph version 4.0 changes

(1) The 'auto' SE flag now makes SE's with even dimensions. e.g. 2x2

(2) The rasterio utilities (in morph/utils) were rewritten to properly

process images with an odd number of pixels per row.

Also, OpenFile now decompresses unix-compressed (*.Z) rasterfiles.

(3) Three new rank-order dependent filters were added,

LUMFilt, LUMSmooth, and LUMSharp.

LUMfilt is the lower-upper-middle filter as defined by

Hardie, R. C., and C. G. Boncelet, "LUM filters: A class of

rank-order-based filters for smoothing and sharpening,"

IEEE Trans. Signal Processing, vol. SP-41, No. 3, March 1993.

LUMSMO is the LUM smoothing filter defined therein, and LUMSHA

is the LUM sharpening filter.

These filters compare the center pixel in a neighborhood defined

by an SE to upper and lower order statistics in the neighborhood.

Depending on the ordering either the center pixel or one of

the order stats is output.

morph version 3.0 changes

There are two major changes in morph v3.0.

(1) morph calls a subroutine called morph_sub which actually performs

the morphology routines.  morph decodes the command line arguments

and performs raster file I/O.  morph_sub accepts pointers to input and

output image arrays and the size of the  image as well as its own control

parameters. This completely decouples the morphology routines from

any file specifications.  It ought to be easy now to incorporate the

morphology routines into any program.

(2) I modified the binary image erode and dilate routines to accept a

"not" flag. This flag causes the erode to pass the input pixel untouched

unless the result is true inwhich case the routine returns a zero.

This permits deletion of specific white features from a binary  image using

a hit-or-miss structuring element (SE) or deletion of white interiors interiors

using standard SE's.

The flag causes the dilate to pass the input pixel untouched

unless the result is false inwhich case the routine returns a one.

The action is analogue applied to black pixels of the erode applied to

white pixels. That is, it permits deletion of specific black features from a

binary  image using a hit-or-miss structuring element (SE) or deletion of

white interiors interiors using standard SE's.

morph version 2.0 changes

There was a bug in the dilation procedures in version 1.1. In brief,

the problem was that the origin of the structuring element was incorrectly

placed so that the output image was shifted incorrectly. This is usually

not a problem. It was not until I did something that depended on the

correct shifting of the output that I noticed the problem. Unfortunately,

fixing the problem required changing code in 6 of the 11 source files

that comprise morph.

Another problem with vers. 1.1 was that the tophat operator did not work.

That was a problem in morph.c. It has been fixed and now works properly.

Source files in /nfs/taacman/rap2/morph/2d changed from vers. 1.1:

mdilate.c

merode.c

mimageio.c

mimgmanip.c

morph.c

mrank.c

Source files unchanged from vers. 1.1:

Makefile

mgetargs.c

mloadse.c

morph.h

mthresh.c

Any place a line has been altered, the following comment has been placed:

v2.0

Here is a detailed explanation of what was wrong and how I fixed it.

Before a dilation is performed, the structuring element (SE) is rotated by

180 degrees around the SE origin. This is a mathematical necessity. (See

either of the papers cited in morph.h for an explanation.) My dilation

routines did rotate the SE 180 degrees, but not with respect to the origin.

instead they rotated the SE within its bounding rectangle and left the

origin unchanged with respect to the rectangle.

Example:  x marks the SE origin, ____ is the SE top, *==== is the SE bottom.

______         =====*         =====*

| x    |       |      |       | x    |

|      |       |      |       |      |

|      |       |    x |       |      |

*=====         ______         ______

(a) original   (b) correct    (c) the way morph 1.1 did it

Note that this caused no error at all when the SE origin was at the center

of the bounding rectangle, as it is for most useful SEs.

This would have been easy to change but for the way that I handled

image boundaries. By default, the program zeropads around the boundaries

of the images to be transformed. That is, it puts borders of zeros around

the perimeter of the image.

The size of these were a function of the SE as follows:

Let lb and rb be the number of columns of zeros in left border

and right border and let tb and bb be the number of rows of zeros in the

top and bottom borders. Let sox and soy be the SE origin coordinates where

(sox,soy) = (0,0) corresponds to the upper left hand corner of the image.

x is horizontal displacement right from that corner , y is vertical

displacement down from the corner. Let sX be the width in pixels of the minimum covering rectangle of the SE. Let sY be the height in pixels of the same.

My program set lb = sox, rb = sX-sox-1, tb = soy, bb = sY-soy-1. This was

fine when the dilation left the SE origin unchanged with respect to the

bounding rectangle. However, correcting the SE origin shift caused any

SE with origin not in the center of the bounding rectangle, to go outside

the defined image.

To correct the problem, I fixed the origin shift in the dilate routines and

I replaced the old borders with rb = lb = sX-1 and tb = bb = sY-1.

To update your old version of morph, I suggest the following:

. ftp to taacman (129.59.100.104)

. cd to /nfs/taacman/users/rap2/morph/2d

. copy all the files in the directory (optionally, you need only copy the five

files with changes)

. make

If you have any questions or comments contact me directly by email or phone.

rap2